Experiment 1: Full Wave Rectifier

COMMON MISTAKES

- DO NOT GROUND THE POINT IN RED.

- If the red point is grounded, it can burn the circuit.

Only this point is to be grounded

DO NOT GROUND THIS POINT

Points to Note –

- Ripple voltage magnitude ~ 20mV.

- In order to see the ripple on the oscilloscope, set the scope on AC Coupling

- Set the Probe value to 1:1

- Set the trigger value to less than 20mV.

Note – Make sure that all the grounds are connected to the same point.

EXPERIMENT 2: Dual Power Supply with Center Tap Transformer

This is the common ground

Attention: This capacitor should have reverse polarity as indicated because  0 is more positive than -ve

Points to Note –

- The capacitor polarity is important here because voltages of opposite polarity are generated by this circuit.

- The top capacitor should have it’s positive (+) connected to “+ve” as shown in the diagram. And negative (-) connected to “0 Volts/GND” as indicated in the circuit.

- The bottom capacitor should have its positive (+) connected to “0 Volts/GND”. And negative (-) connected to “-ve” as shown.

- Note that there is one common ground. The center tap, ends of the resistors and capacitors are all connected to the same common ground.

Common Mistakes –

- The capacitors get burnt when they are not connected as per the polarity shown. So be careful with connecting the capacitors. Ask TA when in doubt.

- The center tap of the transformer is not connected to the common ground which leads to a faulty operation of the circuit. So always connect the center tap to the same ground as the rest of the circuit.